# CIP-010-AB-4

# Cyber Security — Configuration Change Management and Vulnerability Assessments

# Standard Effective Date: October 1, 2024

## Audit Summary

| Registered Entity | [Registered Entity name as it appears in the AESO ARS Registry] |
| --- | --- |
| Functional Entity | [Functional entities for which the Registered Entity above was registered during the audit period] |
| Audit Period | From: [Audit start date or standard effective date, whichever comes later] To: [Audit end date or standard withdrawal/supersede date, whichever comes first] Note: Submitted evidence must demonstrate compliance for the entire audit period. |
| Audit | [Scheduled (YYYY-QX) or Spot Check YYYY-MM-DD] |
| Compliance Monitoring Entity | Alberta Electric System Operator (AESO) |
| Suspected Non-Compliance to the standard? | [If Yes, list the requirements with suspected contravention findings e.g. R1, R2, R5]. |
| Date of Completion | [Use YYYY-MM-DD format] |

## Assessment Commentary

[Information (if any) relevant to audit findings below]

## Findings

| R1 | Suspected Contraventions | AESO Use Only |
| --- | --- | --- |
|  | Areas of Concern and Observations | AESO Use Only |
| R2 | Suspected Contraventions | AESO Use Only |
|  | Areas of Concern and Observations | AESO Use Only |
| R3 | Suspected Contraventions | AESO Use Only |
|  | Areas of Concern and Observations | AESO Use Only |
| R4 | Suspected Contraventions | AESO Use Only |
|  | Areas of Concern and Observations | AESO Use Only |

## Contact Information

| Audited Entity |  |
| --- | --- |
| Compliance Primary | Name: Title: Phone: Email: |
| Subject Matter Expert | Name: Title: Phone: Email: |

| AESO Team |  |
| --- | --- |
| Lead Auditor | Name: Title: Phone: Email: |
| Auditor | Name: Title: Phone: Email: |
| Compliance Manager | Name: Title: Phone: Email: |
| Standard Owner | Name: Title: Phone: Email: |

## Applicability

4.1. Functional Entities: For the purpose of the requirements contained herein, the following list of functional entities will be collectively referred to as “Responsible Entities.” For requirements in this standard where a specific functional entity or subset of functional entities are the applicable entity or entities, the functional entity or entities are specified explicitly.

4.1.1. [Intentionally left blank.]

4.1.2. a legal owner of an electric distribution system that owns one or more of the following facilities, systems, and equipment for the protection or restoration of the bulk electric system:

4.1.2.1. Each underfrequency load shedding or under voltage load shed system that:

4.1.2.1.1. is part of a load shedding program that is subject to one or more requirements in a reliability standard; and

4.1.2.1.2. performs automatic load shedding under a common control system owned by the Responsible Entity, without human operator initiation, of 300 MW or more;

4.1.2.2. Each remedial action scheme where the remedial action scheme is subject to one or more requirements in a reliability standard;

4.1.2.3. Each protection system (excluding underfrequency load shedding and under voltage load shed) that applies to any electric distribution system where the protection system is subject to one or more requirements in a reliability standard; and

4.1.2.4. Each cranking path and group of elements meeting the initial switching requirements from a blackstart resource up to and including the first point of connection of the starting station service of the next generating unit(s) or aggregated generating facility(ies)  to be started.

4.1.3. the operator of a generating unit that is part of the bulk electric system and the operator of an aggregated generating facility that is part of the bulk electric system;

4.1.4. the legal owner of a generating unit that is part of the bulk electric system and the legal owner of an aggregated generating facility that is part of the bulk electric system;

4.1.5. [Intentionally left blank.]

4.1.6. the operator of a transmission facility;

4.1.7. the legal owner of a transmission facility; and

4.1.8. the ISO.

4.2. Facilities: For the purpose of the requirements contained herein, the following facilities, systems, and equipment owned by each Responsible Entity in Section 4.1 above are those to which these requirements are applicable. For requirements in this reliability standard where a specific type of facilities, system, or equipment or subset of facilities, systems, and equipment are applicable, these are specified explicitly.

4.2.1. Legal owner of an electric distribution system and legal owner of a transmission facility: One or more of the following facilities, systems and equipment that operate at, or control elements that operate at, a nominal voltage of 25 kV or less and are owned by a legal owner of an electric distribution system or a legal owner of a transmission facility for the protection or restoration of the bulk electric system:

4.2.1.1. Each underfrequency load shedding or under voltage load shed system that:

4.2.1.1.1. is part of a load shedding program that is subject to one or more requirements in a reliability standard; and

4.2.1.1.2. performs automatic load shedding under a common control system owned by the Responsible Entity, without human operator initiation, of 300 MW or more.

4.2.1.2. Each remedial action scheme where the remedial action scheme is subject to one or more requirements in a reliability standard.

4.2.1.3. Each protection system (excluding underfrequency load shedding and under voltage load shed) that applies to any transmission facility or electric distribution system where the protection system is subject to one or more requirements in a reliability standard.

4.2.1.4. Each cranking path and group of elements meeting the initial switching requirements from a blackstart resource up to and including the first point of connection of the starting station service of the next generating unit(s) or aggregated generating facility(ies) to be started.

4.2.2. Responsible Entities listed in 4.1 other than a legal owner of an electric distribution system:

all bulk electric system facilities.

4.2.3. Exemptions: The following are exempt from CIP-010-AB-4:

4.2.3.1. Cyber assets at facilities regulated by the Canadian Nuclear Safety Commission.

4.2.3.2. Cyber assets associated with communication networks and data communication links between discrete electronic security perimeters.

4.2.3.3. [Intentionally left blank.].

4.2.3.4. For the legal owner of an electric distribution system, the systems and equipment that are not included in section 4.2.1 above.

4.2.3.5. Responsible Entities that identify that they have no BES cyber systems categorized as high impact or medium impact according to the CIP-002 identification and categorization processes.

## Notes on Applicability

- Applicability varies between CIP standards. Be sure to review the applicability for each CIP standard. Differences include but may not be limited to:

  - CIP-005, CIP-010 and CIP-013 include exemption 4.2.3.5, CIP-003 does not.

  - CIP-013 does not include criterion 4.1.2.4 which is present in all other CIP standards.

## Compliance Assessment

| ### Requirement |  |  | ### Measure |  |  |
| --- | --- | --- | --- | --- | --- |
| R1 Each Responsible Entity shall implement one or more documented processes that collectively include each of the applicable requirement parts in CIP‐010-AB‐1 Table R1 – Configuration Change Management. |  |  | M1 Evidence must include each of the applicable documented processes that collectively include each of the applicable requirement parts in CIP‐010-AB‐1 Table R1 – Configuration Change Management and additional evidence to demonstrate implementation as described in the Measures column of the table. |  |  |
| ### Part 1.1 in Table R1 – Configuration Change Management |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; - physical access control systems; and - protected cyber assets Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems - physical access control systems; and - protected cyber assets Requirement Develop a baseline configuration, individually or by group, which shall include the following items: 1.1.1. operating system(s) (including version) or firmware where no independent operating system exists; 1.1.2. any commercially available or open‐source application software (including version) intentionally installed; 1.1.3. any custom software installed; 1.1.4. any logical network accessible ports; and 1.1.5. any security patches applied. |  |  | M1.1 Examples of evidence may include, but are not limited to: - a spreadsheet identifying the required items of the baseline configuration for each cyber asset, individually or by group; or - a record in an asset management system that identifies the required items of the baseline configuration for each cyber asset, individually or by group. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR1.1 Please provide: - One or more documented processes pertaining to R1 Part 1.1. | Provide descriptions for submitted evidence: | Relative link to evidence: | Verify the Responsible Entity has documented one or more processes throughout the entire audit period to develop a baseline configuration for all applicable cyber assets which includes the following items: - operating system(s) (including version) or firmware where no independent operating system exists; - any commercially available or open-source application software (including version) intentionally installed; - any custom software installed; - any logical network accessible ports; and - any security patches applied. | For AESO use only |  |
| - A list of all cyber assets subject to R1 Part 1.1. - Evidence that one or more documented processes pertaining to R1 Parts 1.1.1 through 1.1.5 have been implemented. If there are more than 9 cyber assets which are subject to R1 Part 1.1 in the list in item (ii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication of which groups of cyber assets and time periods each example pertains to. |  |  | Verify the Responsible Entity has developed baseline configurations for all applicable cyber assets individually or by group, which include each of the items in Parts 1.1.1 to 1.1.5, if applicable. Verify each baseline existed throughout the entire audit period, or from the date on which the cyber asset was placed in a production environment to the date on which the cyber asset was removed from a production environment. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R1.1. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 1.2 in Table R1 – Configuration Change Management |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; - physical access control systems; and - protected cyber assets Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems - physical access control systems; and - protected cyber assets Requirement Authorize and document changes that deviate from the existing baseline configuration. |  |  | M1.2 Examples of evidence may include, but are not limited to: - a change request record and associated electronic authorization (performed by the individual or group with the authority to authorize the change) in a change management system for each change; or - documentation that the change was performed in accordance with the requirement. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR1.2 Please provide: - One or more documented processes pertaining to R1 Part 1.2. |  |  | Verify the Responsible Entity has documented one or more processes throughout the entire audit period to authorize and document changes to applicable cyber assets that deviate from the existing baseline configuration. | For AESO use only |  |
| - A list of all cyber assets subject to R1 Part 1.2. - A list of all changes that deviate from the existing baseline configuration. - Evidence that one or more documented processes pertaining to R1 Part 1.2 have been implemented. If there are more than 9 changes in the list in item (iii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. |  |  | Verify the Responsible Entity has authorized and documented all changes to applicable cyber assets that deviate from the existing baseline configuration which occurred during the audit period. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R1.2. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 1.3 in Table R1 – Configuration Change Management |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; - physical access control systems; and - protected cyber assets Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems - physical access control systems; and - protected cyber assets Requirement For a change that deviates from the existing baseline configuration, update the baseline configuration as necessary within 30 days of completing the change. |  |  | M1.3 An example of evidence may include, but is not limited to, updated baseline documentation with a date that is within 30 days of the date of the completion of the change. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR1.3 Please provide: - One or more documented processes pertaining to R1 Part 1.3. |  |  | Verify the Responsible Entity has documented one or more processes throughout the entire audit period to update the baseline configuration of applicable cyber assets as necessary within 30 days of completing any change that deviates from the existing baseline configuration. | For AESO use only |  |
| - A list of all cyber assets subject to R1 Part 1.3. - A list of all changes that deviate from the existing baseline configuration. - Evidence that one or more documented processes pertaining to R1 Part 1.3 have been implemented. If there are more than 9 changes in the list in item (iii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. |  |  | Verify the Responsible Entity has updated the baseline configurations of applicable cyber assets as necessary within 30 days of completing each change that deviates from the existing baseline configuration and has occurred during the audit period. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R1.3. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 1.4 in Table R1 – Configuration Change Management |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; - physical access control systems; and - protected cyber assets Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems - physical access control systems; and - protected cyber assets Requirement For a change that deviates from the existing baseline configuration: 1.4.1. prior to the change, determine required cyber security controls in CIP‐005 and CIP‐007 that could be impacted by the change; 1.4.2. following the change, verify that required cyber security controls determined in 1.4.1 are not adversely affected; and 1.4.3. document the results of the verification. |  |  | M1.4 An example of evidence may include, but is not limited to, a list of cyber security controls verified or tested along with the dated test results. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR1.4 Please provide: - One or more documented processes pertaining to R1 Part 1.4. |  |  | Verify the Responsible Entity has documented one or more processes throughout the entire audit period such that for each change that deviates from the existing baseline the Responsible Entity: - determines, prior to the change, required cyber security controls in CIP-005 and CIP-007 that could be impacted by the change; - verifies, following the change, that required cyber security controls determined in Part 1.4.1 are not adversely affected; and - documents the results of the verification. | For AESO use only |  |
| - A list of all cyber assets subject to R1 Part 1.4. - A list of all changes that deviate from the existing baseline configuration. - Evidence that one or more processes pertaining to R1 Parts 1.4.1 through 1.4.3 have been implemented. If there are more than 9 changes in the list in item (iii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. |  |  | Verify that, for each change to an Applicable System that deviated from the existing baseline configuration and occurred during the audit period, the Responsible Entity has: - determined the required security controls in CIP-005 and CIP-007 that could be impacted by the change prior to making the change; - verified that the required cyber security controls determined in Part 1.4.1, were not adversely affected following the change; and - documented the results of the verification. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R1.4. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 1.5 in Table R1 – Configuration Change Management |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems Requirements Where technically feasible, for each change that deviates from the existing baseline configuration: 1.5.1. prior to implementing any change in the production environment, test the changes in a test environment or test the changes in a production environment where the test is performed in a manner that minimizes adverse effects, that models the baseline configuration to ensure that required cyber security controls in CIP‐005 and CIP‐007 are not adversely affected; and 1.5.2. document the results of the testing and, if a test environment was used, the differences between the test environment and the production environment, including a description of the measures used to account for any differences in operation between the test and production environments. |  |  | M1.5 An example of evidence may include, but is not limited to a list of cyber security controls tested along with successful test results and a list of differences between the production and test environments with descriptions of how any differences were accounted for, including the date of the test. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR1.5 Please provide: - One or more documented processes pertaining to R1 Part 1.5. |  |  | Verify the Responsible Entity has documented one or more processes throughout the entire audit period such that, for changes to applicable cyber assets that deviate from the existing baseline configuration: - prior to implementing any change in the production environment, the Responsible Entity tests the changes in a test environment or tests the changes in a production environment where the test is performed in a manner that minimizes adverse effects, that models the baseline configuration to ensure that required cyber security controls in CIP-005 and CIP-007 are not adversely affected; and - the Responsible Entity documents the results of the testing and, if a test environment was used, the differences between the test environment and the production environment, including a description of the measures used to account for any differences in operation between the test and production environments. | For AESO use only |  |
| - A list of all cyber assets subject to R1 Part 1.5. - A list of all changes that deviate from the existing baseline configuration. - Evidence that one or more documented processes pertaining to R1 Part 1.5 have been implemented. If there are more than 9 changes in the list in item (iii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. |  |  | Verify that, for each change to an Applicable System that deviated from the existing baseline configuration and occurred during the audit period, prior to implementing any change in the production environment, the Responsible Entity has tested the changes in: - a test environment; or - a production environment where the test was performed in a manner that minimizes adverse effects, that models the baseline configuration to ensure that required cyber security controls in CIP-005 and CIP-007 are not adversely affected. Verify that, for each change to an Applicable System that deviated from the existing baseline configuration and occurred during the audit period, the Responsible Entity has documented: - the results of the testing; and - the differences between the test environment and the production environment, including a description of the measures used to account for any differences in operation between the test and production environments, if a test environment was used. | For AESO use only |  |
| - A list of all approved Technical Feasibility Exceptions applicable to this Part, if any, and evidence of implementation of all their terms and conditions. |  |  | Verify the terms and conditions of each approved Technical Feasibility Exception have been implemented. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R1.5. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 1.6 in Table R1 – Configuration Change Management |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; and - physical access control systems Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems; and - physical access control systems Note: Implementation does not require the Responsible Entity to renegotiate or abrogate existing contracts (including amendments to master agreements and purchase orders). Additionally, the following issues are beyond the scope of Part 1.6: (1) the actual terms and conditions of a procurement contract; and (2) vendor performance and adherence to a contract. Requirements Prior to a change that deviates from the existing baseline configuration associated with baseline items in Parts 1.1.1, 1.1.2, and 1.1.5, and when the method to do so is available to the Responsible Entity from the software source: 1.6.1. Verify the identity of the software source; and 1.6.2. Verify the integrity of the software obtained from the software source. |  |  | M1.6 An example of evidence may include, but is not limited to a change request record that demonstrates the verification of identity of the software source and integrity of the software was performed prior to the baseline change or a process which documents the mechanisms in place that would automatically ensure the identity of the software source and integrity of the software. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR1.6 Please provide: - One or more documented processes pertaining to R1 Part 1.6. |  |  | Verify the Responsible Entity has documented one or more processes for all applicable cyber assets throughout the entire audit period that include: - Verification of the identity of the software source; and - Verification of the integrity of the software obtained from the software source; or - Documentation of the mechanisms in place that would automatically ensure the identity of the software source and integrity of the software, if applicable. | For AESO use only |  |
| - A list of all cyber assets subject to R1 Part 1.6. - A list of changes that deviate from the existing baseline configuration associated with baseline items in Parts 1.1.1, 1.1.2, and 1.1.5. - Evidence that one or more documented processes pertaining to R1 Part 1.6 have been implemented. If there are more than 9 changes in the list in item (iii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. - Where no method to verify the identity of the software source (Part 1.6.1) or to verify the integrity of the software from the software source (Part 1.6.2) is available to the Responsible Entity, evidence to support or corroborate that there is no such method available. |  |  | Verify that prior to each change to an Applicable System that deviates from the existing baseline configuration associated with baseline items in Parts 1.1.1, 1.1.2, and 1.1.5 and has occurred during the audit period: - The Responsible Entity has verified the identity of the software source; and - The Responsible Entity has verified the integrity of the software obtained from the software source. If the Responsible Entity stated that there was no method to verify the identity of one or more software sources or to verify the integrity of the software from one or more software sources, verify the submitted evidence supports the Responsible Entity’s assertion. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R1.6. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |

Suspected Contraventions

AESO Use Only

Areas of Concern and Observations

AESO Use Only

| ### Requirement |  |  | ### Measure |  |  |
| --- | --- | --- | --- | --- | --- |
| R2 Each Responsible Entity shall implement one or more documented processes that collectively include each of the applicable requirement parts in CIP‐010-AB‐1 Table R2 – Configuration Monitoring. |  |  | M2 Evidence must include each of the applicable documented processes that collectively include each of the applicable requirement parts in CIP‐010-AB‐1 Table R2 – Configuration Monitoring and additional evidence to demonstrate implementation as described in the Measures column of the table. |  |  |
| ### Part 2.1 in Table R2 – Configuration Monitoring |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; and - protected cyber assets Requirement Monitor at least once every 35 days for changes to the baseline configuration (as described in requirement R1, Part 1.1). Document and investigate detected unauthorized changes. |  |  | M2.1 An example of evidence may include, but is not limited to, logs from a system that is monitoring the configuration along with records of investigation for any unauthorized changes that were detected. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR2.1 Please provide: - One or more documented processes pertaining to R2 Part 2.1. | Provide descriptions for AR1 (i) submitted evidence: | Relative link to evidence: | - Verify the Responsible Entity has documented one or more processes throughout the entire audit period to: - monitor at least once every 35 days for changes to the baseline configuration (as described in R1, Part 1.1), and - document and investigate detected unauthorized changes. | For AESO use only |  |
| - A list of all cyber assets subject to R2 Part 2.1. - A list of all changes to the baseline configuration (as described in R1, Part 1.1). - A list of all detected unauthorized changes. - Evidence that the processes pertaining to R2 Part 2.1 have been implemented. If there are more than 9 cyber assets in the list in item (ii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to | Provide descriptions for AR1 (i) submitted evidence: | Relative link to evidence: | - Verify the Responsible Entity monitored all applicable cyber assets at least once every 35 days throughout the entire audit period for changes to the baseline configuration (as described in R1, Part 1.1). - Verify all detected unauthorized changes were documented and investigated. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R2.1. | Provide descriptions for other submitted evidence: | Relative link to evidence: | - Review any other evidence provided. | For AESO use only |  |

Suspected Contraventions

AESO Use Only

Areas of Concern and Observations

AESO Use Only

| ### Requirement |  |  | ### Measure |  |  |
| --- | --- | --- | --- | --- | --- |
| R3 Each Responsible Entity shall implement one or more documented processes that collectively include each of the applicable requirement parts in CIP‐010-AB‐1 Table R3– Vulnerability Assessments. |  |  | M3 Evidence must include each of the applicable documented processes that collectively include each of the applicable requirement parts in CIP‐010-AB‐1 Table R3 – Vulnerability Assessments and additional evidence to demonstrate implementation as described in the Measures column of the table. |  |  |
| ### Part 3.1 in Table R3 – Vulnerability Assessments |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; - physical access control systems; and - protected cyber assets Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems - physical access control systems; and - protected cyber assets Requirement At least once every 15 months, conduct a paper or active vulnerability assessment. |  |  | M3.1 Examples of evidence may include, but are not limited to: - a document listing the date of the assessment (performed at least once every 15 months), the controls assessed for each BES cyber system along with the method of assessment; or - a document listing the date of the assessment and the output of any tools used to perform the assessment. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR3.1 Please provide: - One or more documented processes pertaining to R3 Part 3.1. | Provide descriptions for AR1 (i) submitted evidence: | Relative link to evidence: | Verify the Responsible Entity has documented one or more processes throughout the entire audit period for conducting a paper or active vulnerability assessment at least once every 15 months. | For AESO use only |  |
| - A list of all cyber assets subject to R3 Part 3.1. - Evidence that one or more processes pertaining to R3 Part 3.1 have been implemented. | Provide descriptions for AR1 (i) submitted evidence: | Relative link to evidence: | Verify the Responsible Entity has conducted a paper or active vulnerability assessment on all applicable cyber assets at least once every 15 months throughout the entire audit period. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R3.1. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 3.2 in Table R3 – Vulnerability Assessments |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems Requirement Where technically feasible, at least once every 36 months: 3.2.1 perform an active vulnerability assessment in a test environment, or perform an active vulnerability assessment in a production environment where the test is performed in a manner that minimizes adverse effects, that models the baseline configuration of the BES cyber system in a production environment; and 3.2.2 document the results of the testing and, if a test environment was used, the differences between the test environment and the production environment, including a description of the measures used to account for any differences in operation between the test and production environments. |  |  | M3.2 An example of evidence may include, but is not limited to, a document listing the date of the assessment (performed at least once every 36 months), the output of the tools used to perform the assessment, and a list of differences between the production and test environments with descriptions of how any differences were accounted for in conducting the assessment. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR3 Part 3.2 Please provide: - One or more documented processes pertaining to R3 Part 3.2. |  |  | - Verify the Responsible Entity has documented one or more processes throughout the entire audit period to: - perform an active vulnerability assessment in a test environment, or perform an active vulnerability assessment in a production environment where the test was performed in a manner that minimized adverse effects, that modelled the baseline configuration of the BES cyber system in a production environment at least once every 36 months; and - document the results of the testing and, if a test environment was used, the differences between the test environment and the production environment, including a description of the measures used to account for any differences in operation between the test and production environments. | For AESO use only |  |
| - A list of all cyber assets subject to R3 Part 3.2. - Evidence that one or more processes pertaining to R3 Part 3.2 have been implemented. |  |  | - Verify that the Responsible Entity has: - for all applicable cyber assets, performed an active vulnerability assessment in a test environment, or performed an active vulnerability assessment in a production environment where the test was performed in a manner that minimized adverse effects, that modelled the baseline configuration of the BES cyber system in a production environment at least once every 36 months; and - documented the results of the testing and, if a test environment was used, the differences between the test environment and the production environment, including a description of the measures used to account for any differences in operation between the test and production environments. | For AESO use only |  |
| - A list of all approved Technical Feasibility Exceptions applicable to this Part, if any, and evidence of implementation of all their terms and conditions. |  |  | - Verify that the terms and conditions of each approved Technical Feasibility Exception have been implemented. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R3.2. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 3.3 in Table R3 – Vulnerability Assessments |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; and - protected cyber assets Requirement Prior to adding a new applicable cyber asset to a production environment, perform an active vulnerability assessment of the new cyber asset, except for CIP exceptional circumstances and like replacements of the same type of cyber asset with a baseline configuration that models an existing baseline configuration of the previous or other existing cyber asset. |  |  | M3.3 An example of evidence may include, but is not limited to, a document listing the date of the assessment (performed prior to the commissioning of the new cyber asset) and the output of any tools used to perform the assessment. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR3.3 Please provide: - One or more documented processes pertaining to R3 Part 3.3. |  |  | Verify the Responsible Entity has documented one or more processes throughout the entire audit period for performing an active vulnerability assessment of each new applicable cyber asset, prior to adding the cyber asset to a production environment, except for CIP exceptional circumstances and like replacements of the same type of cyber asset with a baseline configuration that models an existing baseline configuration of the previous or other existing cyber asset. | For AESO use only |  |
| - A list of new applicable cyber assets being added to a production environment during the audit period, an indication of whether they are subject to R3 Part 3.3, and if they are not subject to R3 Part 3.3, an indication of whether a CIP exceptional circumstance applied or whether this was a like replacement. - Evidence that one or more documented processes pertaining to R3 Part 3.3 have been implemented. If there are more than 9 cyber assets in the list in item (ii) which are subject to R3 Part 3.3, an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. |  |  | - Verify that an active vulnerability assessment of each applicable cyber asset has been performed prior to adding it to a production environment, except for CIP exceptional circumstances and like replacements of the same type of cyber asset with a baseline configuration that models an existing baseline configuration of the previous or other existing cyber asset. | For AESO use only |  |
| - If a CIP exceptional circumstance is applicable to any of the cyber assets in the list in item (iii), the rationale why it was applied. - If any of the cyber assets in the list in item (iii) were like replacements of the same type of cyber asset, evidence that the baseline configuration of the new cyber asset models an existing baseline configuration of the previous or other existing cyber asset. |  |  | Validate the rationale for applying CIP exceptional circumstance is appropriate and the Responsible Entity has adhered to any applicable cyber security policies. For like replacement of the same type of cyber asset, verify the baseline configuration of the new cyber asset models an existing baseline configuration of the previous or other existing cyber asset. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R3.2. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Part 3.4 in Table R1 – Vulnerability Assessments |  |  | ### Measure |  |  |
| Applicable Systems High Impact BES cyber systems and their associated: - electronic access control or monitoring systems; - physical access control systems; and - protected cyber assets Medium Impact BES cyber systems and their associated: - electronic access control or monitoring systems - physical access control systems; and - protected cyber assets Requirement Document the results of the assessments conducted according to Parts 3.1, 3.2, and 3.3 and the action plan to remediate or mitigate vulnerabilities identified in the assessments including the planned date of completing the action plan and the execution status of any remediation or mitigation action items. |  |  | M3.4 An example of evidence may include, but is not limited to, a document listing the results or the review or assessment, a list of action items, documented proposed dates of completion for the action plan, and records of the status of the action items (such as minutes of a status meeting, updates in a work order system, or a spreadsheet tracking the action items). |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR3.4 Please provide: - One or more documented processes pertaining to R3 Part 3.4. |  |  | - Verify the Responsible Entity has documented one or more processes throughout the entire audit period to document: - the results of the assessments conducted according to Parts 3.1, 3.2, and 3.3; and - the action plan to remediate or mitigate vulnerabilities identified in the assessments including the planned date of completing the action plan and the execution status of any remediation or mitigation action items. | For AESO use only |  |
| - A list of all cyber assets subject to R3 Part 3.4. - A list of all assessments conducted according to Parts 3.1, 3.2, and 3.3. - A list of all action plans to remediate or mitigate vulnerabilities identified in the assessments, if any and the assessments to which they pertain. - Evidence that the processes pertaining to R3 Part 3.4 have been implemented. If there are more than 9 action plans in the list in item (iii) an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. If different formats are used for the action plans for different groups of cyber assets or different time periods, an example of each format must be provided with an indication regarding which groups of cyber assets and time periods each example pertains to. |  |  | - For each assessment conducted according to Parts 3.1, 3.2, and 3.3 during the audit period, verify the Responsible Entity has documented the result of the assessment. - For each assessment conducted according to Parts 3.1, 3.2, and 3.3 during the audit period, verify the Responsible Entity has documented the action plan to remediate or mitigate the identified vulnerabilities, if any, including: - a planned date of completing the action plan; and - the execution status of any remediation or mitigation action items. | For AESO use only |  |
| or any other evidence to demonstrate compliance with R3.4. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |

Suspected Contraventions

AESO Use Only

Areas of Concern and Observations

AESO Use Only

| ### Requirement |  |  | ### Measure |  |  |
| --- | --- | --- | --- | --- | --- |
| R4 Each Responsible Entity, for its high impact and medium impact BES cyber systems and associated protected cyber assets, shall implement, except under CIP exceptional circumstances, one or more documented plan(s) for transient cyber assets and removable media that include the sections in Attachment 1. Responsible Entities shall include each of the sections provided below in their plan(s) for transient cyber asset(s) and removable media as required under Requirement R4 |  |  | M4 Evidence shall include each of the documented plan(s) for transient cyber assets and removable media that collectively include each of the applicable sections in Attachment 1 and additional evidence to demonstrate implementation of plan(s) for transient cyber assets and removable media. Additional examples of evidence per section are located in Attachment 2. If a Responsible Entity does not use transient cyber assets and removable media, examples of evidence include, but are not limited to, a statement, policy, or other document that states the Responsible Entity does not use transient cyber assets and removable media. |  |  |
| ### Section 1. Transient Cyber Asset(s) Managed by the Responsible Entity (Attachment 1) |  |  | ### Measure |  |  |
| 1.1. Transient Cyber Asset Management Responsible Entities shall manage transient cyber assets, individually or by group: (1) in an ongoing manner to ensure compliance with applicable requirements at all times, (2) in an on- demand manner applying the applicable requirements before connection to a BES cyber system, or (3) a combination of both (1) and (2) above. 1.2. Transient Cyber Asset Authorization For each individual or group of transient cyber assets, each Responsible Entity shall authorize: 1.2.1. Users, either individually or by group or role; 1.2.2. Locations, either individually or by group; and 1.2.3. Uses, which shall be limited to what is necessary to perform business functions. 1.3. Software Vulnerability Mitigation Use one or a combination of the following methods to achieve the objective of mitigating the risk of vulnerabilities posed by unpatched software on the transient cyber asset (per transient cyber asset capability): - Security patching, including manual or managed updates; - Live operating system and software executable only from read-only media; - System hardening; or - Other method(s) to mitigate software vulnerabilities. 1.4. Introduction of Malicious Code Mitigation Use one or a combination of the following methods to achieve the objective of mitigating the introduction of malicious code (per transient cyber asset capability): - Antivirus software, including manual or managed updates of signatures or patterns; - Application allowlisting; or - Other method(s) to mitigate the introduction of malicious code. 1.5. Unauthorized Use Mitigation Use one or a combination of the following methods to achieve the objective of mitigating the risk of unauthorized use of transient cyber assets: - Restrict physical access; - Full-disk encryption with authentication; - Multi-factor authentication; or - Other method(s) to mitigate the risk of unauthorized use. |  |  | Section 1.1 Examples of evidence for Section 1.1 may include, but are not limited to, the method(s) of management for the transient cyber assets. This can be included as part of the transient cyber asset plan(s), part of the documentation related to authorization of transient cyber assets managed by the Responsible Entity or part of a security policy. Section 1.2 Examples of evidence for Section 1.2 may include, but are not limited to, documentation from asset management systems, human resource management systems, or forms or spreadsheets that show authorization of transient cyber assets managed by the Responsible Entity. Alternatively, this can be documented in the overarching plan document. Section 1.3 Examples of evidence for Section 1.3 may include, but are not limited to, documentation of the method(s) used to mitigate software vulnerabilities posed by unpatched software such as security patch management implementation, the use of live operating systems from read-only media, system hardening practices or other method(s) to mitigate the software vulnerability posed by unpatched software. Evidence can be from change management systems, automated patch management solutions, procedures or processes associated with using live operating systems, or procedures or processes associated with system hardening practices. If a transient cyber asset does not have the capability to use method(s) that mitigate the risk from unpatched software, evidence may include documentation by the vendor or Responsible Entity that identifies that the transient cyber asset does not have the capability. Section 1.4 Examples of evidence for Section 1.4 may include, but are not limited to, documentation of the method(s) used to mitigate the introduction of malicious code such as antivirus software and processes for managing signature or pattern updates, application allowlisting practices, processes to restrict communication, or other method(s) to mitigate the introduction of malicious code. If a transient cyber asset does not have the capability to use method(s) that mitigate the introduction of malicious code, evidence may include documentation by the vendor or Responsible Entity that identifies that the transient cyber asset does not have the capability. Section 1.5 Examples of evidence for Section 1.5 may include, but are not limited to, documentation through policies or procedures of the method(s) to restrict physical access; method(s) of the full-disk encryption solution along with the authentication protocol; method(s) of the multi-factor authentication solution; or documentation of other method(s) to mitigate the risk of unauthorized use. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR 4.1 Please provide: - One or more documented plan(s) for transient cyber asset and removable media that address Section 1 in Attachment 1 | Provide descriptions for submitted evidence: | Relative link to evidence: | - Verify the Responsible Entity has documented one or more documented plan(s) for transient cyber assets and removable media throughout the entire audit period that address Section 1 in Attachment 1. | For AESO use only |  |
| - A list of transient cyber assets (TCA) which are managed by the Responsible Entity that include the following information: - TCA identifier - TCA management approach (ongoing or on-demand) - Description of the TCA’s purpose - BES cyber assets, protected cyber assets and electronic security perimeters where the TCA is used - Evidence that the plans pertaining to Section 1 in Attachment 1 have been implemented. If there are more than 9 transient cyber assets in the list in item (ii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. - Where a transient cyber asset does not have the capability to mitigate the risk from unpatched software or the capability to use methods that mitigate the introduction of malicious code, evidence to support or corroborate that the transient cyber asset does not have these capabilities. |  |  | - Section 1.1 - Verify that throughout the entire audit period the Responsible Entity has implemented its plan(s) to manage transient cyber assets individually or by group: - (1) in an ongoing manner to ensure compliance with applicable requirements at all times; - (2) in an on-demand manner applying the applicable requirements before connection to a BES cyber system; or - (3) a combination of both (1) and (2) above. - Section 1.2 - For each sampled transient cyber asset, verify that, throughout the entire audit period, the Responsible Entity authorized: - Users, either individually or by group or role; - locations, either individually or by group; and - uses, which were limited to what was necessary to perform business functions. - Section 1.3 - Verify that, throughout the entire audit period, the Responsible Entity has implemented one or a combination of the following methods to achieve the objective of mitigating the risk of vulnerabilities posed by unpatched software on each sampled transient cyber asset (per transient cyber asset capability): - security patching, including manual or managed updates; - live operating system and software executable only from read-only media; - system hardening; or - other method(s) to mitigate software vulnerabilities. - If the Responsible Entity stated that a sampled transient cyber asset does not have the capability to use method(s) that mitigate the risk from unpatched software, verify the submitted evidence supports the Responsible Entity’s assertion. - Section 1.4 - Verify that, throughout the entire audit period, the Responsible Entity has implemented one or a combination of the following methods to achieve the objective of mitigating the introduction of malicious code (per transient cyber asset capability): - antivirus software, including manual or managed updates of signatures or patterns; - application allowlisting; or - other method(s) to mitigate the introduction of malicious code. - If the Responsible Entity stated that a sampled transient cyber asset does not have the capability to use methods that mitigate the introduction of malicious code, verify the submitted evidence supports the Responsible Entity’s assertion. - Section 1.5 - Verify that the Responsible Entity has implemented one or a combination of the following methods to achieve the objective of mitigating the risk of unauthorized use of each sampled transient cyber asset: - restrict physical access; - full-disk encryption with authentication; - multi-factor authentication; or - other method(s) to mitigate the risk of unauthorized use. | For AESO use only |  |
| or any other evidence to demonstrate compliance with Section 1 in Attachment 1. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | For AESO use only |  |
| ### Section 2. Transient Cyber Asset(s) Managed by a Party Other than the Responsible Entity. (Attachment 1) |  |  | ### Measure |  |  |
| 2.1. Software Vulnerabilities Mitigation Use one or a combination of the following methods to achieve the objective of mitigating the risk of vulnerabilities posed by unpatched software on the transient cyber asset (per transient cyber asset capability): - Review of installed security patch(es); - Review of security patching process used by the party; - Review of other vulnerability mitigation performed by the party; or - Other method(s) to mitigate software vulnerabilities. 2.2. Introduction of malicious code mitigation Use one or a combination of the following methods to achieve the objective of mitigating malicious code (per transient cyber asset capability): - Review of antivirus update level; - Review of antivirus update process used by the party; - Review of application allowlisting used by the party; - Review use of live operating system and software executable only from read-only media; - Review of system hardening used by the party; or - Other method(s) to mitigate malicious code. 2.3. For any method used to mitigate software vulnerabilities or malicious code as specified in 2.1 and 2.2, Responsible Entities shall determine whether any additional mitigation actions are necessary and implement such actions prior to connecting the transient cyber asset. |  |  | Section 2.1 Examples of evidence for Section 2.1 may include, but are not limited to, documentation from change management systems, electronic mail or procedures that document a review of installed security patch(es); memoranda, electronic mail, policies or contracts from parties other than the Responsible Entity that identify the security patching process or vulnerability mitigation performed by the party other than the Responsible Entity; evidence from change management systems, electronic mail, system documentation or contracts that identifies acceptance by the Responsible Entity that the practices of the party other than the Responsible Entity are acceptable; or documentation of other method(s) to mitigate software vulnerabilities for transient cyber assets managed by a party other than the Responsible Entity. If a transient cyber asset does not have the capability to use method(s) that mitigate the risk from unpatched software, evidence may include documentation by the Responsible Entity or the party other than the Responsible Entity that identifies that the transient cyber asset does not have the capability. Section 2.2 Examples of evidence for Section 2.2 may include, but are not limited to, documentation from change management systems, electronic mail or procedures that document a review of the installed antivirus update level; memoranda, electronic mail, system documentation, policies or contracts from the party other than the Responsible Entity that identify the antivirus update process, the use of application allowlisting, use of live of operating systems or system hardening performed by the party other than the Responsible Entity; evidence from change management systems, electronic mail or contracts that identifies the Responsible Entity’s acceptance that the practices of the party other than the Responsible Entity are acceptable; or documentation of other method(s) to mitigate malicious code for transient cyber assets managed by a party other than the Responsible Entity. If a transient cyber asset does not have the capability to use method(s) that mitigate the introduction of malicious code, evidence may include documentation by the Responsible Entity or the party other than the Responsible Entity that identifies that the transient cyber asset does not have the capability. Section 2.3 Examples of evidence for Section 2.3 may include, but are not limited to, documentation from change management systems, electronic mail, or contracts that identifies a review to determine whether additional mitigations are necessary and that they have been implemented prior to connecting the transient cyber asset managed by a party other than the Responsible Entity. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR 4.2 Please provide: - One or more documented plan(s) for transient cyber assets and removable media that address Section 2 in Attachment 1 |  |  | - Verify the Responsible Entity has documented one or more documented plan(s) for transient cyber assets and removable media throughout the entire audit period that address Section 2 in Attachment 1. | - For AESO use only |  |
| - A list of transient cyber assets (TCA) which are managed by a party other than the Responsible Entity that include the following information: - TCA identifier - Description of the TCA’s purpose - BES cyber assets, protected cyber assets and electronic security perimeters where the TCA is used - Evidence that the plans pertaining to Section 2 in Attachment 1 have been implemented. If there are more than 9 transient cyber assets in the list in item (ii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. - Where a transient cyber asset does not have the capability to mitigate the risk from unpatched software or the capability to use methods that mitigate the introduction of malicious code, evidence to support or corroborate that the transient cyber asset does not have these capabilities. |  |  | Section 2.1 Verify that, throughout the entire audit period, the Responsible Entity has implemented one or a combination of the following methods to achieve the objective of mitigating the risk of vulnerabilities posed by unpatched software on the sampled transient cyber assets (per transient cyber asset capability): - review of installed security patch(es); - review of security patching process used by the party; - review of other vulnerability mitigation performed by the party; or - other method(s) to mitigate software vulnerabilities. - If the Responsible Entity stated that a sampled transient cyber asset does not have the capability to use method(s) that mitigate the risk from unpatched software, verify the submitted evidence supports the Responsible Entity’s assertion. Section 2.2 Verify that, throughout the entire audit period, the Responsible Entity has implemented one or a combination of the following methods to achieve the objective of mitigating malicious code on the sampled transient cyber assets (per transient cyber asset capability): - review of antivirus update level; - review of antivirus update process used by the party; - review of application allowlisting used by the party; - review use of live operating system and software executable only from read-only media; - review of system hardening used by the party; or - other method(s) to mitigate malicious code. - If the Responsible Entity stated that a sampled transient cyber asset does not have the capability to use methods that mitigate the introduction of malicious code, verify the submitted evidence supports the Responsible Entity’s assertion. Section 2.3 For any method used to mitigate software vulnerabilities or malicious code as specified in 2.1 and 2.2: - Verify that the Responsible Entity determined whether any additional mitigation actions are necessary. - If any additional mitigation actions were necessary, verify that such actions were implemented prior to connecting the transient cyber asset. | - For AESO use only |  |
| or any other evidence to demonstrate compliance with Section 2 in Attachment 1. | Provide descriptions for other submitted evidence: | Relative link to evidence: | Review any other evidence provided. | - For AESO use only |  |
| ### Section 3. Removable Media (Attachment 1) |  |  | ### Measure |  |  |
| 3.1 Removable Media Authorization For each individual or group of removable media, each Responsible Entity shall authorize: 3.1.1. Users, either individually or by group or role; and 3.1.2. Locations, either individually or by group. 3.2. Malicious Code Mitigation To achieve the objective of mitigating the threat of introducing malicious code to high impact or medium impact BES cyber systems and their associated protected cyber assets, each Responsible Entity shall: 3.2.1. Use method(s) to detect malicious code on removable media using a cyber asset other than a BES cyber system or protected cyber assets; and 3.2.2. Mitigate the threat of detected malicious code on removable media prior to connecting the removable media to a high impact or medium impact BES cyber system or associated protected cyber assets. |  |  | - Section 3.1 - Examples of evidence for Section 3.1 may include, but are not limited to, documentation from asset management systems, human resource management systems, forms or spreadsheets that shows authorization of removable media. The documentation must identify removable media, individually or by group of removable media, along with the authorized users, either individually or by group or role, and the authorized locations, either individually or by group. - Section 3.2 - Examples of evidence for Section 3.2 may include, but are not limited to, documented process(es) of the method(s) used to mitigate malicious code such as results of scan settings for removable media, or implementation of on-demand scanning. Documented process(es) for the method(s) used for mitigating the threat of detected malicious code on removable media, such as logs from the method(s) used to detect malicious code that show the results of scanning and that show mitigation of detected malicious code on removable media or documented confirmation by the entity that the removable media was deemed to be free of malicious code. |  |  |
| ### Evidence Submission | ### Evidence Description | ### Evidence | ### Assessment Approach | ### Auditor Notes |  |
| AR4.3 Please provide: - One or more documented plan(s) for transient cyber assets and removable media that address Section 3 in Attachment 1 |  |  | - Verify the Responsible Entity has documented one or more documented plan(s) for transient cyber assets and removable media throughout the entire audit period that address Section 3 in Attachment 1. | - For AESO use only |  |
| - A list of removable media (RM) that include the following information: - RM identifier - A list of the instances when the RM was connected to a BES cyber asset or protected cyber asset - Description of the RM’s purpose - BES cyber assets and protected cyber assets s where the RM is used - Evidence that the plans pertaining to Section 3 in Attachment 1 have been implemented. If there are more than 9 removable media in the list in item (ii), an example of the implementation may be provided instead, and the auditor will sample for additional implementation evidence. |  |  | - Section 3.1 - Verify that, throughout the entire audit period, the Responsible Entity authorized, for each sampled removable media: - users, either individually or by group or role; and - locations, either individually or by group. - Section 3.2 - Verify, throughout the entire audit period, the Responsible Entity has implemented the following methods to achieve the objective of mitigating the threat of introducing malicious code to high impact or medium impact BES cyber systems and their associated protected cyber assets: - use method(s) to detect malicious code on each sampled removable media using a cyber asset other than a BES cyber system or protected cyber asset; and - mitigate the threat of detected malicious code on each sampled removable media prior to connecting the removable media to a high impact or medium impact BES cyber system or associated protected cyber assets. | - For AESO use only |  |
| or any other evidence to demonstrate compliance with Section 3 in Attachment 1. | Provide descriptions for other submitted evidence: | Relative link to evidence: | - Review any other evidence provided. | - For AESO use only |  |

Suspected Contraventions

AESO Use Only

Areas of Concern and Observations

AESO Use Only

## General Notes

The AESO developed this Reliability Standard Audit Worksheet (RSAW) to add clarity and consistency to the audit team’s assessment of compliance with this reliability standard, including the approach elected to assess requirements.

Additionally, the RSAW provides a non-exclusive list of examples of the types of evidence a market participant may produce or may be asked to produce to demonstrate compliance with this reliability standard. A market participant’s adherence to the examples contained within this RSAW does not constitute compliance with the reliability standard.

This document is not an AESO authoritative document and revisions to it may be made from time to time by the AESO. Market participants are notified of revisions through the stakeholder update process.

## Notes to File

[For AESO use only: any observations, remarks or action items for future audits]

## Revision History

| Version | Issue Date | Description |
| --- | --- | --- |
| 1.0 | 2023-02-23 | Initial version |
| 1.1 | 2023-04-26 | Removed “and associated cyber assets” from lists in R1. |
| 1.2 | 2023-09-25 | Minor formatting changes to align with latest CIP/COM RSAWs |