PRC-023-AB-2
Transmission Relay Loadability

# Audit Summary

| Registered Entity: | [Registered Entity name as it appears in the AESO ARS Registry] |  |
| --- | --- | --- |
| Functional Entity: | [Functional entities for which the Registered Entity above was registered throughout the audit period] |  |
| Audit Period: | From: [Audit start date or standard effective date, whichever comes later] To: [Audit end date or standard withdrawal/supersede date, whichever comes first] |  |
| Audit: | [Scheduled (YYYY-QX) or Spot Check YYYY-MM-DD] |  |
| Compliance Monitoring Entity: | Alberta Electric System Operator (AESO) |  |
| Suspected Non-Compliance to the standard? |  |  |
|  |  | [If Yes, list the requirements with suspected contravention findings e.g. R1, R2, R7] |
| Date of Completion: | [Use YYYY-MM-DD format] |  |

## Assessment Commentary

[Information (if any) relevant to audit findings below]

## Findings

| R1 | [Summary of Findings] |
| --- | --- |
| R2 | [Summary of Findings] |
| R3 | [Summary of Findings] |
| R4 | [Summary of Findings] |
| R5 | [Summary of Findings] |

# Contact Information

| Audited Entity |  |  |
| --- | --- | --- |
| Compliance Primary | [Name] [Title] [Phone] [Email] |  |
| Subject Matter Expert | [Name] [Title] [Phone] [Email] |  |
| AESO Team |  | Sign-off |
| Lead Auditor | [Name] [Title] [Phone] [Email] | Date: Signature: |
| Auditor | [Name] [Title] [Phone] [Email] | Date: Signature: |
| Compliance Manager | [Name] [Title] [Phone] [Email] | Date: Signature: |
| Standard Owner | [Name] [Title] [Phone] [Email] | Date: Signature: |

# Applicability

This reliability standard applies to:

- a legal owner of a transmission facility with load-responsive phase protection systems, as described in Appendix 1 applied to any one (1) or more of the following facilities:

- transmission lines operated at two hundred (200) kV and above;

- transmission lines operated below two hundred (200) kV which the ISO identifies as essential to the reliability of the bulk electric system as required in requirement R6.2;

- transformers with low voltage terminals connected at two hundred (200) kV and above; or

- transformers with low voltage terminals connected below two hundred (200) kV which the ISO identifies in accordance with requirement R6.2;

- a legal owner of a generating unit, that also owns the associated switch yard, with load-responsive phase protection systems, as described in Appendix 1 applied to any one (1) or more of the following facilities:

- transmission lines operated at two hundred (200) kV and above;

- transmission lines operated below two hundred (200) kV which the ISO identifies as essential to the reliability of the bulk electric system as required in requirement R6.2;

- transformers with low voltage terminals connected at two hundred (200) kV and above; or

- transformers with low voltage terminals connected below two hundred (200) kV which the ISO identifies in accordance with requirement R6.2;

- a legal owner of an aggregated generating facility, that also owns the associated switch yard, with load-responsive phase protection systems, as described in Appendix 1 applied to any one (1) or more of the following facilities:

- transmission lines operated at two hundred (200) kV and above;

- transmission lines operated below two hundred (200) kV which the ISO identifies as essential to the reliability of the bulk electric system as required in requirement R6.2;

- transformers with low voltage terminals connected at two hundred (200) kV and above; or

- transformers with low voltage terminals connected below two hundred (200) kV which the ISO identifies in accordance with requirement R6.2; and

- the ISO.

# Compliance Assessment

## R1

| # Requirement & Measure | Evidence Submission | Evidence Description | Evidence | Assessment Approach | Auditor Notes |
| --- | --- | --- | --- | --- | --- |
| R1 Each legal owner of a transmission facility, legal owner of a generating unit and legal owner of an aggregated generating facility must use one of the criteria set out in requirements R1.1 through R1.14, inclusive, for each specific circuit terminal to prevent its phase protective relay settings from limiting transmission system loadability while maintaining reliable protection of the bulk electric system for all fault conditions and evaluate the phase protective relay’s loadability at zero point eight five (0.85) per unit voltage and a power factor angle of thirty (30) degrees. MR1 Evidence of using one of the criteria set out in requirements R1.1 through R1.14 as required in requirement R1 exists. Evidence may include: (a) spreadsheets or summaries of calculations to show that each of its transmission relays is set in accordance with one of the criteria set out in requirements R1.1 through R1.14; and (b) coordination curves or summaries of calculations that show that relays set per criterion set out in requirement R1.11 do not expose the transformer to fault levels and durations beyond those indicated in the reliability standard. | - AR1 Please provide: | [Click and edit to enter description for AR1(i) submitted evidence] | [Click and edit to embed file or link to evidence] | Review the evidence provided at AR1(i) for appropriateness. | [For AESO use only] |
|  | (i) the list of protective relays determined as specified in Appendix 1; this list should include the relay name, relay device number (e.g. 21, 59), protected facility name, voltage rating of protected facility and criterion (e.g. R1.1, R1.2, etc.) used as basis for evaluating the phase protective relay’s loadability; |  |  |  |  |
|  | (ii) evidence demonstrating that each applicable phase protective relay is set in accordance with one of the criteria set out in requirements R1.1 through R1.14, e.g., spreadsheets, summaries of calculations, coordination curves, etc.; this evidence should identify the parameters and input data in calculating the relay reach (Zrelay30). | [Click and edit to enter description for AR1(ii) submitted evidence. Please insert the evidence and its description in the fields provided below for the appropriate criterion.] | [Click and edit to embed file or link to evidence] | - Review AR1(ii) submitted documentation to determine whether: (a) the audited entity evaluated the phase protective relay’s loadability at zero point eight five (0.85) per unit voltage and a power factor angle of thirty (30) degrees; and (b) the submitted documentation demonstrate that the audited entity set the protective relay so it does not operate at or below the loading conditions as specified in R1.1 through R1.14. | [For AESO use only] |
| R1.1 Set transmission line relays so they do not operate at or below one hundred and fifty (150 %) percent of the highest seasonal facility rating of a circuit for the available defined loading duration nearest to four (4) hours, expressed in amperes; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.2 Set transmission line relays so they do not operate at or below one hundred and fifteen (115 %) percent of the highest seasonal 15-minute facility rating of a circuit, expressed in amperes; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.3 Set transmission line relays so they do not operate at or below one hundred and fifteen (115%) percent of the maximum theoretical power transfer capability, using a ninety (90) degree angle between the sending-end and receiving-end voltages and either reactance or complex impedance of the circuit, expressed in amperes, using one of the following to perform the power transfer calculation: R1.3.1 an infinite source, i.e. zero source impedance, with a one point zero zero (1.00) per unit bus voltage at each end of the transmission line; or R1.3.2 an impedance at each end of the transmission line, which reflects the actual system source impedance with a one point zero five (1.05) per unit voltage behind each source impedance; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.4 Set transmission line relays on series compensated transmission lines so they do not operate at or below the maximum power transfer capability of the transmission line, determined as the greater of: (a) one hundred and fifteen (115 %) percent of the highest emergency rating of the series capacitor, or (b) one hundred and fifteen (115%) percent of the maximum power transfer capability of the circuit, expressed in amperes, calculated in accordance with requirement R1.3, using the full transmission line inductive reactance; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.5 Set transmission line relays on weak source systems so they do not operate at or below one hundred and seventy (170%) percent of the maximum end-of-line three-phase fault magnitude, expressed in amperes; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.6 Set transmission line relays applied on transmission lines connected to a generating unit or aggregated generating facility remote to load so they do not operate at or below two hundred and thirty (230%) percent of the total nameplate capability of all the generating units at the facility; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.7 Set transmission line relays applied at the load center terminal, remote from a generating unit or aggregated generating facility, so they do not operate at or below one hundred and fifteen (115%) percent of the maximum current flow from the load to the generation source under any system configuration; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.8 Set transmission line relays applied on the system-end of transmission lines that serve load remote to the system so they do not operate at or below one hundred and fifteen (115%) percent of the maximum current flow from the system to the load under any system configuration; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.9 Set transmission line relays applied on the load-end of transmission lines that serve load remote to the system so they do not operate at or below one hundred and fifteen (115%) of the maximum current flow from the load to the system under any system configuration; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.10 Set transformer fault protection relays and transmission line relays on transmission lines terminated only with a transformer so that they do not operate at or below the greater of: (a) one hundred and fifty (150%) percent of the applicable maximum transformer nameplate rating, expressed in amperes, including the forced cooled ratings corresponding to all installed supplemental cooling equipment; or (b) one hundred and fifteen (115%) percent of the highest established emergency transformer rating; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.11 Set load responsive transformer fault protection relays, if used, such that the protection settings do not expose the transformer to a fault level and duration that exceeds the transformer’s mechanical withstand capability; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.12 For transformer overload protection relays that do not comply with requirement R1.10: (a) set the relays to allow the transformer to be operated at an overload level of at least one hundred and fifty (150%) percent of the maximum applicable nameplate rating, or one hundred and fifteen (115%) percent of the highest emergency transformer rating, whichever is greater; (b) the protection relay must allow overload in subsection 1.12(a) for at least fifteen (15) minutes to allow the ISO to take controlled action to relieve the overload; (c) install supervision for the relays using either a top oil or simulated winding hot spot temperature element; and (d) the relay setting should be no less than one hundred (100°C) degrees Celsius for the top oil or one hundred and forty (140°C) degrees Celsius for the winding hot spot temperature; |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.13 When the desired transmission line capability is limited by the requirement to adequately protect the transmission line, set the transmission line distance relays to a maximum of one hundred and twenty five (125%) percent of the apparent impedance, at the impedance angle of the transmission line, subject to the following constraints: R1.13.1 Set the maximum torque angle to ninety (90) degrees or the highest setting supported by the manufacturer; R1.13.2 Evaluate the relay loadability in amperes at the relay trip point at zero point eight five (0.85) per unit voltage and a power factor angle of thirty (30) degrees; and R1.13.3 Include a relay setting component of eighty seven (87%) percent of the current calculated in requirement R1.13.2 in the facility rating determination for the circuit; and |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |
| R1.14 Where other situations present practical limitations on circuit capability, set the phase protection relays so they do not operate at or below one hundred and fifteen (115%) percent of such limitations. |  | [Click and edit to enter description, as appropriate.] | [Click and edit to embed file or link to evidence, as appropriate.] |  | [For AESO use only] |

Findings

[For AESO use only]

## R2

| # Requirement & Measure | Evidence Submission | Evidence Description | Evidence | Assessment Approach | Auditor Notes |
| --- | --- | --- | --- | --- | --- |
| R2 Each legal owner of a transmission facility, legal owner of a generating unit and legal owner of an aggregated generating facility must set its out-of-step blocking elements to allow tripping of phase protective relays for faults that occur during the loading conditions used to verify transmission line relay loadability per requirement R1. MR2 Evidence of setting its out-of-step blocking elements as required in requirement R2 exists. Evidence may include spreadsheets or summaries of calculations. | - AR2 Please provide: | [Click and edit to enter description for AR2(i) submitted evidence] | [Click and edit to embed file or link to evidence] | - Review the AR2 submitted documentation to determine whether the audited entity set its out-of-step blocking elements to allow tripping of phase protective relays for faults that occur during the loading conditions used to verify transmission line relay loadability per requirement R1. | [For AESO use only] |
|  | (i) evidence to demonstrate that the out-of-step blocking elements do not block tripping for faults during the loading conditions used to verify transmission line relay loadability per requirement R1, e.g., spreadsheets or summaries of calculations; |  |  |  |  |
|  | or any other evidence to demonstrate compliance with the requirement R2. | [Click and edit to enter description for any other submitted evidence] | [Click and edit to embed file or link to evidence] |  |  |

Findings

[For AESO use only]

## R3

| # Requirement & Measure | Evidence Submission | Evidence Description | Evidence | Assessment Approach | Auditor Notes |
| --- | --- | --- | --- | --- | --- |
| R3 Any legal owner of a transmission facility, legal owner of a generating unit or legal owner of an aggregated generating facility that uses a circuit capability with the practical limitations described in requirements R1.6, R1.7, R1.8, R1.9, R1.13, or R1.14 must use the calculated circuit capability as the facility rating of the circuit and must obtain the agreement of the ISO to use the calculated circuit capability. MR3 Evidence of using, and obtaining the agreement to use, the calculated circuit capability as required in requirement R3 exists. Evidence may include: (a) facility rating spreadsheets or facility rating database to show that the calculated circuit capability was used as the facility rating of the circuit; and (b) dated correspondence to show that the ISO agreed to the calculated circuit capability. | - AR3 Please provide: | [Click and edit to enter description for AR3(i) submitted evidence] | [Click and edit to embed file or link to evidence] | Review the AR3(i) submitted documentation to determine whether the audited entity used the calculated circuit capability that was agreed by the ISO as the facility rating of the circuit. | [For AESO use only] |
|  | (i) evidence to demonstrate that for the practical limitations referenced in R3, if any, the calculated circuit capability is used as the facility rating of the circuit e.g., facility rating spreadsheets, facility rating database; |  |  |  |  |
|  | (ii) evidence to demonstrate that the agreement of the ISO to use the calculated circuit capability as the facility rating was obtained, e.g., dated correspondence. | [Click and edit to enter description for AR3(ii) submitted evidence] | [Click and edit to embed file or link to evidence] | - Review the AR3(ii) submitted documentation and assess whether the audited entity obtained the agreement of the ISO to use the calculated circuit capability. | [For AESO use only] |

Findings

[For AESO use only]

## R4

| # Requirement & Measure | Evidence Submission | Evidence Description | Evidence | Assessment Approach | Auditor Notes |
| --- | --- | --- | --- | --- | --- |
| R4 Any legal owner of a transmission facility, legal owner of a generating unit or legal owner of an aggregated generating facility that uses requirement R1.2 as the basis for verifying transmission line relay loadability must provide the ISO with an updated list of circuits associated with those transmission line relays at least once each calendar year, with no more than fifteen (15) months between reports. MR4 Evidence of providing the ISO with an updated list of circuits as required in requirement R4 exists. Evidence may include email or mail to the appropriate ISO recipient with the updated list which may either be a full list, a list of incremental changes to the previous list, or a statement that there are no changes to the previous list. | - AR4 Please provide: | [Click and edit to enter description for AR4 submitted evidence] | [Click and edit to embed file or link to evidence] | - Review the AR4 submitted documentation to determine whether the audited entity provided the ISO with updates at least once each calendar year, with no more than fifteen (15) months between reports. | [For AESO use only] |
|  | for each instance where the requirement R1.2 was invoked, dated evidence to demonstrate providing the ISO with an updated list, e.g. email or mail to the appropriate ISO recipient with the updated list which may either be: - a full list, - a list of incremental changes to the previous list, or - a statement that there are no changes to the previous list, - or any other evidence to demonstrate compliance with requirement R4. |  |  |  |  |

Findings

[For AESO use only]

## R5

| # Requirement & Measure | Evidence Submission | Evidence Description | Evidence | Assessment Approach | Auditor Notes |
| --- | --- | --- | --- | --- | --- |
| R5 Any legal owner of a transmission facility, legal owner of a generating unit or legal owner of an aggregated generating facility that uses requirement R1.13 as the basis for verifying transmission line relay loadability must provide the ISO with an updated list of circuits associated with those transmission line relays at least once each calendar year, with no more than fifteen (15) months between reports. MR5 Evidence of providing the ISO with an updated list of circuits as required in requirement R5 exists. Evidence may include email or mail to the appropriate ISO recipient with the updated list which may either be a full list, a list of incremental changes to the previous list, or a statement that there are no changes to the previous list. | - AR5 Please provide: | [Click and edit to enter description for AR5(i) submitted evidence] | [Click and edit to embed file or link to evidence] | - Review the submitted documentation as identified in AR5(i), to determine whether the audited entity provided the ISO with the updated list at least once each calendar year, with no more than fifteen (15) months between reports. | [For AESO use only] |
|  | for each instance where the requirement R1.13 was invoked, dated evidence to demonstrate providing the ISO with an updated list, e.g. email or mail to the appropriate ISO recipient with the updated list which may either be: - a full list, - a list of incremental changes to the previous list, or - a statement that there are no changes to the previous list, - or any other evidence to demonstrate compliance with requirement R5. |  |  |  |  |

Findings

[For AESO use only]

# General Notes

The AESO developed this Alberta Reliability Standard Audit Worksheet (RSAW) to add clarity and consistency to the audit team’s assessment of compliance with this reliability standard, including the approach elected to assess requirements.

Additionally, the RSAW provides a non-exclusive list of examples of the types of evidence a market participant may produce or may be asked to produce to demonstrate compliance with the Alberta Reliability Standard. A market participant’s adherence to the examples contained within this RSAW does not constitute compliance with the applicable Alberta Reliability Standard.

This document is not an AESO authoritative document and revisions to it may be made from time to time by the AESO. Market participants are notified of revisions through the stakeholder update process.

# Notes to File

[For AESO use only: any observations, remarks or action items for future audits]

# Revision History

| Version | Issue Date | Description |
| --- | --- | --- |
| 1.0 | May 16, 2013 | Initial version of Worksheet |
| 2.0 | December 17, 2015 | Revised template and content |